S1

-------

interface vlan 3

ip address 192.168.3.11 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.3.1

interface range f0/2 - 4, f0/7 - 24, g0/1 - 2

switchport mode access

switchport access vlan 7

shutdown

interface f0/6

switchport mode access

switchport access vlan 3

no shutdown

interface f0/1

switchport mode trunk

switchport trunk native vlan 8

switchport trunk allowed vlan 3,4,8

interface f0/5

switchport mode trunk

switchport trunk native vlan 8

switchport trunk allowed vlan 3,4,8

exit

exit

copy running-config startup-config

S2

----------

interface vlan 3

ip address 192.168.3.12 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.3.1

interface range f0/2 - 17, f0/19 - 24, g0/1 - 2

switchport mode access

switchport access vlan 7

shutdown

interface f0/18

switchport mode access

switchport access vlan 4

no shutdown

interface f0/1

switchport mode trunk

switchport trunk native vlan 8

switchport trunk allowed vlan 3,4,8

interface f0/5

switchport mode trunk

switchport trunk native vlan 8

switchport trunk allowed vlan 3,4,8

-------------

R1

-------------

enable

config terminal

interface g0/0/1

no shutdown

exit

interface g0/0/1.3

description Management Network

encapsulation dot1q 3

ip address 192.168.3.1 255.255.255.0

interface g0/0/1.4

description Operations Network

encapsulation dot1q 4

ip address 192.168.4.1 255.255.255.0

interface g0/0/1.8

description Native VLAN

encapsulation dot1q 8 native