# TUTORIAL 3.1: VLAN and Inter-VLAN Routing

Question 1 b) (Continued)

- The newly added university’s campus network topology is shown in Figure 1-2. The VLAN information is shown in Table 1-2. As for now, SW1 and SW3 have been configured. You are required to suggest configurations for SW2 and R1 based on the requirements needed.

Requirements:

  - Use the VLAN information shown in Table 1-2.

  - Both VLANs are able to communicate.

  - Allow all VLANs in the trunk links.

Figure 1-2: Campus Network Topology Table 1-2: VLAN Information

| VLAN ID | VLAN Name | Network Address |
| --- | --- | --- |
| VLAN 10 | Lecturer | 192.168.10.0 /25 |
| VLAN 15 | Admin | 192.168.10.128 /25 |

(11 marks)

| SW2 | vlan 10 name Lecturer vlan 15 name Admin int fa0/1 switchport mode access switchport access vlan 10 int fa0/2 switchport mode access switchport access vlan 15 int range g0/1-2 switchport mode trunk switchport trunk allowed vlan 10,15 |
| --- | --- |
| R1 | int g0/0.10 encapsulation dot1q 10 ip address 192.168.10.1 255.255.255.128 int g0/0.15 encapsulation dot1q 15 ip address 192.168.10.129 255.255.255.128 int g0/0 no shut |