# Tutorial 5.1: EtherChannel

- A new cable has been attached between SW1 and SW2, as shown in Figure 2-2a, to form an EtherChannel. However, after configuration, the EtherChannel does not form. Figure 2-2b shows the partial EtherChannel configurations for SW1 and SW2. Assume that the configurations on SW1 are correct.

Figure 2-2a: An EtherChannel Network

| SW1#show running-configuration Building configuration… … interface Port-channel1 Switchport mode trunk ! interface FastEthernet0/1 Switchport mode trunk Duplex full Channel-group 1 mode passive ! interface FastEthernet0/2 Switchport mode trunk Duplex full Channel-group 1 mode passive | SW2#show running-configuration Building configuration… … interface Port-channel1 Switchport mode access ! interface FastEthernet0/3 Switchport mode access Duplex half Channel-group 1 mode passive ! interface FastEthernet0/4 Switchport mode access Duplex half Channel-group 1 mode passive |
| --- | --- |

Figure 2-2b: Partial EtherChannel configurations for SW1 and SW2

  - Identify the type of EtherChannel that is being used.	(1 mark)

LACP passive

  - Identify errors in SW2 configurations.	(3 marks)

- The switchport mode for interface FastEthernet0/3 and FastEthernet0/4 should be set as trunk mode instead of access mode. - The duplex settings of interface FastEthernet0/3 and FastEthernet0/4 on SW2 (Half duplex) do not match with the interface FastEthernet0/1 and FastEthernet0/2 on SW1 (Full duplex). - Since the SW1’s interfaces are using passive mode, the etherchannel group will not be formed if SW2’s interfaces are also applying passive mode.

  - Suggest the solutions and configurations to the answers you mentioned in Question 2 b)

(ii).	(6 marks)

- The switchport mode on both interfaces FastEthernet0/3 and FastEthernet0/4 in SW2 should be set as trunk mode. - int range f0/3-4 - switchport mode trunk - The duplex on both interfaces FastEthernet0/3 and FastEthernet0/4 in SW2 should be set as full duplex. - int range f0/3-4 - duplex full - The port-channel 1 mode in SW2 should be switched to active mode. - int range f0/3-4 - no channel-group - channel-group 1 mode active