# Tutorial 6: FHRP Concepts

- Explain the purpose of First Hop Redundancy Protocols (FHRP).

Mechanisms that provide alternate default gateways in switched networks where two or more routers are connected to the same VLANs.

- List FHRP options in a production environment.

- Hot Standby Router (HSRP) - HSRP for IPv6 - Virtual Router Redundancy Protocol version 2 (VRRPv2) - VRRPv3 - gateway Load Balancing Protocol (GLBP) - GLBP for IPv6 - ICMP Router Discovery Protocol (IRDP)

- Explain Hot Standby Router Protocol (HSRP).

- HSRP is a Cisco-proprietary FHRP that is designed to allow for transparent failover of a first-hop IPv4 device. - HSRP is used in a group of routers for selecting an active device and a standby device.

- Discuss how an active router is decided within a group of routers configured with HSRP.

- The active router will be decided based on their HSRP priority, the router with the highest HSRP priority will be elected as active router. - If all routers have the same priority, the router with the higher IPv4 address will be elected as active router.

- Explain the steps taken by a standby router if an active router fails if HSRP is configured.

- The standby device will replace the active router.

- Describe the purpose of HSRP preemption. What will happen if HSRP preemption is not configured?

- HSRP preemption is the ability of an HSRP router to trigger the re-election process. - When preemption is enabled, the router that comes online with a higher HSRP priority will be assume the role of the active router. - Preemption only allows a router to become the active router if it has a higher priority. - Without HSRP preemption, the active router will continue to be active even if a router with a higher priority is added. The active role will only be transferred from one to another when the previously elected active router is down.

- Briefly describe HSRP states.

- Initial - This state is entered through a configuration change or when an interface first becomes available. - Learn - The router has not determined the virtual IP address and has not yet seen a hello message from the active router. In this state, the router waits to hear from the active router. - Listen - The router knows the virtual IP address, but the router is neither the active router nor the standby router. It listens for hello messages from those routers. - Speak - The router sends periodic hello messages and actively participates in the election of the active and / or standby router. - Standby - The router is a candidate to become the next active router and sends periodic hello messages. - Active - The router won the election.

- With reference to Figure 6.1 and Table 6.1, answer the following questions.

Figure 6.1: Network configured with HSRP

| R1 | R2 |
| --- | --- |
| R1#configure terminal R1(config)#interface gi0/1 R1(config-if)#ip address 10.1.1.2 255.255.255.0 R1(config-if)#standby version 2 R1(config-if)#standby 1 ip 10.1.1.1 R1(config-if)#standby 1 priority 150 R1(config-if)#standby 1 preempt | R2#configure terminal R2(config)#interface gi0/1 R2(config-if)#ip address 10.1.1.3 255.255.255.0 R2(config-if)#standby version 2 R2(config-if)#standby 1 ip 10.1.1.1 R2(config-if)#standby 1 priority 110 |

Table 6.1: HSRP Configuration

  - What is the IP address assign to R1 g0/1?

10.1.1.2/24

  - What is the IP address assign to R2 g0/1?

10.1.1.3/24

  - What is the virtual IP address for standby group 1 in R1?

10.1.1.1

  - What is the virtual IP address for standby group 1 in R2?

10.1.1.1

  - Is the virtual IP address for standby group 1 in R1 and R2 must be same?

Yes.

  - What is the default gateway IP address for PC1?

10.1.1.1

  - What is the default gateway IP address for PC2?

10.1.1.1

  - Explain “standby 1 priority 150”.

The router with priority 150 will be elected as active router as it is the highest priority compared to other routers.

  - Explain “standby 1 preempt”.

The router has enabled the HSRP preemption which can trigger re-election process. When a router with higher priority is added, the re-election process will be triggered and elect it to become the active router.

- You are given the basic configuration of two routers (Router A and Router B) in a network that are set up for High Availability using Hot Standby Router Protocol (HSRP) as shown in Table

6.2. Answer the following questions based on Table 6.2.

| Router A | Router B |
| --- | --- |
| interface GigabitEthernet0/0 ip address 192.168.1.2 255.255.255.0 standby 1 ip 192.168.1.1 standby 1 priority 110 standby 1 preempt | interface GigabitEthernet0/0 ip address 192.168.1.3 255.255.255.0 standby 1 ip 192.168.1.1 standby 1 priority 100 |

Table 6.2 HSRP configuration for Router A and Router B

- Identify the active router.

Router A

- Justify your answer in Question 9 a).

Router A has the higher HSRP priority compared to Router B.

- Identify the virtual router’s IPv4 address.

192.168.1.1

- Explain the purpose of the preempt keyword seen in the configuration for Router A.

To trigger the re-election process so that the router with the highest priority will be elected as active router dynamically.