# Tutorial 6.1: FHRP

c) With reference to Figure 2-3a and Figure 2-3b, HSRP have not been fully configured on R2. Write a configuration on R2 to enable VLAN10 as backup and VLAN20 as primary with priority 105. Remove unnecessary configurations.

Figure 2-3a: A HSRP Networ

| R1#show running-configuration Building configuration… … interface FastEthernet0/0 no ip address ! interface FastEthernet0/0.10 encapsulation dot1Q 10 ip address 192.168.10.1 255.255.255.0 standby 1 ip 192.168.10.3 standby 1 priority 105 standby 1 preempt ! interface FastEthernet0/0.20 encapsulation dot1Q 20 ip address 192.168.20.2 255.255.255.0 standby 2 ip 192.168.20.4 | R2#show running-configuration Building configuration… … interface FastEthernet0/0 no ip address ! interface FastEthernet0/0.10 encapsulation dot1Q 10 ip address 192.168.10.2 255.255.255.0 standby 3 preempt ! interface FastEthernet0/0.20 encapsulation dot1Q 20 ip address 192.168.20.3 255.255.255.0 standby 4 preempt |
| --- | --- |

Figure 2-3b: Partial HSRP configurations for R1 and R2

(4 marks)

int FastEthernet0/0.10 no standby 3 preempt standby 1 ip 192.168.10.3 int FastEthernet0/0.20 no standby 4 preempt standby 2 ip 192.168.20.4 standby 2 priority 105 standby 2 preempt