# Tutorial 9: Switch Security Configuration

- The simplest and most effective method to prevent MAC address table overflow attacks is to enable port security. Explain port security.

- Limit the number of MAC address allowed on particular switch port. - By default, it dynamically learns a limited number of MAC address. - It will shutdown the port when exceeded the maximum MAC address number.

- Explain the steps to mitigate a VLAN hopping attack.

- Step 1: Disable DTP (auto trunking) negotiations on non-trunking ports by using the switchport mode access interface configuration command. - Step 2: Disable unused ports and put them in an unused VLAN - Step 3: Manually enable the trunk link on a trunking port by using the switchport mode trunk command. - Step 4: Disable DTP (auto trunking) negotiations on trunking ports by using the switchport nonegotiate command. - Step 5: Set the native VLAN to a VLAN other than VLAN 1 by using the switchport trunk native vlan vlan_number command.

- The network attackers can manipulate the Spanning Tree Protocol (STP) to conduct an attack by spoofing the root bridge and changing the topology of a network. How to mitigate a STP attack?

- PortFast - PortFast immediately brings a port to the forwarding state from a blocking state, bypassing the listening and learning states. - Apply to all end-user access ports. - BPDU Guard - BPDU guard immediately error disables a port that receives a BPDU. - Like PortFast, BPDU guard should only be configured on interfaces attached to end devices.

- There are some port security configuration errors depicted in Figure 9.1. Interface f0/1 should be configured using sticky secure MAC addresses. In addition, the port is assigned to VLAN 20. The requirement is to set the maximum address to 5 and when violation occurred, the switch would drop unknown MAC address and a security notification is not presented by the switch. Document your answer using Table 9.1 in your answer sheet.

Switch(config-if)#int f0/1

Switch(config-if)#switchport mode access Switch(config-if)#switchport port-security Switch(config-if)#switchport port-security max 6

Switch(config-if)#switchport port-security violation shutdown

Figure 9.1: Configuration for sticky secure MAC address

| Error Configurations | Solutions |
| --- | --- |
| The interface f0/1 is not assigned to VLAN 20. | switchport access vlan 20 |
| Incorrect maximum number of address allowed stated. | switchport port-security max 6 |
| Incorrect violation action set. | switchport port-security violation protect |
| The current state does not remember devices’ MAC address. | switchport port-security mac-address sticky |

Table 9.1: Documentation Table

- As a network administrator, you are tasked with implementing port security on a Cisco switch in your organisation's network. Provide the full port security configurations to meet the following requirements for the interface F0/6 of Switch1.

  - It belongs to VLAN22

  - It learns and adds the MAC address to the running-configuration file

  - It limits up to 2 MAC addresses allowed

  - The interface would transition into error-disabled state immediately and generates a syslog message

interface f0/6 switchport mode access switchport access vlan 22 switchport port-security switchport port-security mac-address sticky switchport port-security maximum 2 switchport port-security violation shutdown