# Tutorial 9.1 Switch Security Configuration

Question 4 (Continued)

- The ABC network topology is shown in Figure 4-2. Provide answers to the questions below:

Figure 4-2: ABC Network Topology

(i) Explain DHCP Snooping.	(2 marks)

- DHCP Snooping filters DHCP messages and rate-limits DHCP traffic on untrusted ports, preventing DHCP spoofing attacks. - Devices under administrative control are trusted sources such as switches, routers and servers. - Devices outside the network and all access ports are generally treated as untrusted sources. Tutor answer: - Mitigate DHCP spoofing attack - Filter DHCP messages and limit DHCP traffic on untrusted port - Untrusted ports: Fa0/1 and Fa0/2 - Trusted ports: Fa0/23 and Fa0/24

(ii) Provide DHCP Snooping configuration according to Figure 4-2 with a rate limit of 6.

(3 marks)

ip dhcp snooping interface range f0/23-24 ip dhcp snooping trust exit interface range f0/1-2 ip dhcp snooping limit rate 6 exit ip dhcp snooping vlan 1 end