CCNAv7 ENSA Skills Assessment

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 R1 R1 | G0/0/0 | 10.67.254.2 | 255.255.255.252 | N/A |
|  | G0/0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | Lo0 | 10.52.0.1 | 255.255.255.248 | N/A |
| R2 | G0/0/0 | 10.67.254.1 | 255.255.255.252 | N/A |
| R2 | G0/0/1 | 10.67.1.1 | 255.255.255.0 | N/A |
| R2 | Lo0 | 209.165.201.1 | 255.255.255.224 | N/A |
| S1 | VLAN 1 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| S2 | VLAN 1 | 10.67.1.2 | 255.255.255.0 | 10.67.1.1 |

Assessment Objectives

Part 1: Initialize, Reload and Configure Basic Device Settings (40 points, 40 minutes)

Part 2: Configure and Single Area OSPFv2 (20 points, 10 minutes)

Part 3: Optimize Single Area OSPFv2 (25 points, 20 minutes)

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 1 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

Part 4: Configure Access Control, NAT, and perform configuration backup (15 points, 30 minutes)

Scenario

In this Skills Assessment (SA) you will configure the devices in a small network. You must configure a router,  switch and PCs to support IPv4 connectivity for supported hosts. Your router and switch must also be  managed securely. You will configure Single-Area OSPFv2, NAT, and access control lists. Further, you will  backup up your working configurations to a TFTP server.

Required Resources

• 2 Routers (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable) • 2 Switches (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable) • 2 PCs (Windows with a terminal emulation program, such as Tera Term)

• Console cables to configure the Cisco IOS devices via the console ports

• Ethernet cables as shown in the topology

Instructions

Part 1: Initialize, Reload and Configure Basic Device Settings

Total points: 40

Time: 40 minutes

Step 1: Initialize and reload routers and switches.

Erase the startup configurations and VLANs from the router and switch and reload the devices. Before proceeding, ask your instructor verify device initializations.

Step 2: Configure the routers.

Configuration tasks for R1 and R2 include the following:

| Task | Specification | R1 | R2 |
| --- | --- | --- | --- |
| Disable DNS lookup |  | 0.5 pt | 0.5 pt |
| Router name | R1 or R2, as appropriate | 0.5 pt | 0.5 pt |
| Domain name | ccna-lab.com | 0.5 pt | 0.5 pt |
| Encrypted privileged EXEC password | ciscoenpass | 0.5 pt | 0.5 pt |
| Console access password | ciscoconpass | 0.5 pt | 0.5 pt |
| Set the minimum length for passwords | 10 characters | 0.5 pt | 0.5 pt |
| Create an administrative user in the local database | Username: admin Password: admin1pass | 0.5 pt | 0.5 pt |
| Set login on VTY lines to use local database |  | 0.5 pt | 0.5 pt |
| Set VTY lines to accept SSH connections only |  | 0.5 pt | 0.5 pt |
| Encrypt the clear text passwords |  | 0.5 pt | 0.5 pt |

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 2 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

| Task | Specification | R1 | R2 |
| --- | --- | --- | --- |
| Configure an MOTD Banner |  | 0.5 pt | 0.5 pt |
| Configure interface G0/0/1 | Set the description Set the Layer 3 IPv4 address Activate Interface | 2 pts | 2 pts |
| Configure interface G0/0/0 | Set the description Set the Layer 3 IPv4 address Activate Interface | 2 pts | 2 pts |
| Configure interface Lo0 | Configure IPv4 address | 0.5 pt | 0.5 pts |
| Generate an RSA crypto key | 1024 bits modulus | 0.5 pt | 0.5 pt |

Step 3: Configure S1 and S2.

Configuration tasks for the switches include the following:

| Task | Specification | S1 | S2 |
| --- | --- | --- | --- |
| Disable DNS lookup |  | 0.5 pt | 0.5 pt |
| Switch name | S1 or S2, as appropriate | 0.5 pt | 0.5 pt |
| Domain name | ccna-lab.com | 0.5 pt | 0.5 pt |
| Encrypted privileged EXEC password | ciscoenpass | 0.5 pt | 0.5 pt |
| Console access password | ciscoconpass | 0.5 pt | 0.5 pt |
| Shutdown all unused interfaces |  | 0.5 pt | 0.5 pt |
| Create an administrative user in the local database | Username: admin Password: admin1pass | 0.5 pt | 0.5 pt |
| Set login on VTY lines to use local database |  | 0.5 pt | 0.5 pt |
| Set VTY lines to accept SSH connections only |  | 0.5 pt | 0.5 pt |
| Encrypt the clear text passwords |  | 0.5 pt | 0.5 pt |
| Configure an MOTD Banner |  | 0.5 pt | 0.5 pt |
| Generate an RSA crypto key | 1024 bits modulus | 0.5 pt | 0.5 pt |
| Configure Management Interface (SVI) for VLAN 1 (the Management VLAN) | Set the Layer 3 IPv4 address | 0.5 pt | 0.5 pt |
| Configure Default Gateway |  | 0.5 pt | 0.5 pt |

Points for Step 1 (5 points):

Enter score here.

Points for Step 2 (21 points):

Enter score here.

Points for Step 3 (14 points):

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 3 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

Enter score here.

Instructor Sign-off Part 1:

Instructor Sign-off

Total points for Part 1 (40 points):

Enter score here.

Part 2: Configure Single Area OSPFv2

Total points: 10

Time: 10 minutes

Configuration tasks for R1 and R2 include the following:

| Task | Specification | R1 | R2 |
| --- | --- | --- | --- |
| Configure the OSPF routing process | Use process id 1 | 1 pt | 1 pt |
| Manually configure the router id | Use 0.0.0.1 for R1 and 0.0.0.2 for R2 | 1 pt | 1 pt |
| Configure network statements | Configure a network statement for each locally attached network using a wild card mask that matches each network’s subnet mask Note: R2 Lo0 network should not be included in the OSPF process. | 3 pts | 3 pts |

Instructor Sign-off Part 2

Instruction Sign-off

Total Points (10 points):

Enter score here.

Part 3: Optimize Single-Area OSPFv2

Total points: 20

Time: 20 minutes

Step 1: Configure R1.

Configuration Tasks for R1 include the following:

| Task | Specification | Points |
| --- | --- | --- |
| Configure passive interfaces | Configure all interfaces that are not directly connected to an OSPF neighbor to be passive | 3 points |
| Configure the reference bandwidth | Adjust the reference bandwidth to 1 Gigabit | 2 points |
| Configure Loopback 0 to report the mask it is configured with instead of a host mask | Configure Loopback0 as a point-to-point network for OSPF | 2 points |
| Tune the timers for your network | Configure the hello time for 30 seconds | 1 point |

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 4 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

Step 2: Configure R2.

Configuration tasks for R2 include the following:

| Task | Specification | Points |
| --- | --- | --- |
| Configure passive interfaces | Configure all interfaces that are not directly connected to an OSPF neighbor to be passive | 2 points |
| Configure the reference bandwidth | Adjust the reference bandwidth to 1 Gigabit | 2 points |
| Provide default routing for the OSPF domain | Configure a static default route with loopback 0 as the exit interface, then share the default information with other OSPF speakers | 5 points |
| Tune the timers for your network | Configure the hello time for 30 seconds | 1 point |
| Tune the DR/BDR election to favor R2 | Set the OSPF priority for R2 to a value of 50 | 2 points |

Points for Step 1 (8 points)

Enter score here.

Points for Step 2 (12 points)

Enter score here.

Instructor Sign-off Part 3:

Instructor Sign-off

Total points (20 points):

Enter score here.

Part 4: Configure Access Control, NAT, and perform configuration backup Total points: 30

Time: 30 minutes

Step 1: Configure host computers.

Configure the host computers PC-A and PC-B with IPv4 addresses. (4 points)

| Description | PC-A | PC-B |
| --- | --- | --- |
| IP Address | 192.168.1.50 | 10.67.1.50 |
| Subnet Mask | 255.255.255.0 | 255.255.255.0 |
| Default Gateway | 192.168.1.1 | 10.67.1.1 |

After configuring each host computer, perform the following tests: (4 points)

| Source | Target | Protocol | Expected Result |
| --- | --- | --- | --- |
| PC-A | PC-B | Ping | Success |
| PC-A | https://209.165.201.1 | HTTPS | Success |
| PC-A | 209.165.201.1 | SSH | Success |

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 5 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

| Source | Target | Protocol | Expected Result |
| --- | --- | --- | --- |
| PC-B | 209.165.201.1 | SSH | Success |

If you get different results, troubleshoot your OSPF and host configurations.

Note: If you are unable to access 209.165.201.1 via https, enter ip http secure-server at R2 CLI. R2(config)# ip http secure-server

Step 2: Configure Access Control on R2.

Create and apply an access control list on R2 named R2-SECURITY to do the following:

| Task | Specification | Points |
| --- | --- | --- |
| Create an access control list | R2-SECURITY | 2 points |
| Control HTTP and HTTPS traffic | Only hosts from the 10.0.0.0/8 network are allowed to reach the web server at 209.165.201.1 | 2 points |
| Control SSH traffic | SSH is not allowed to the address 209.165.201.1 | 2 points |
| Permit traffic | All other traffic, regardless of protocol, is allowed | 2 points |
| Apply the ACL | Filter traffic originating from R1 | 2 points |

After configuring and applying the ACL, perform the following tests: (2 points)

| Source | Target | Protocol | Expected Result |
| --- | --- | --- | --- |
| PC-A | PC-B | Ping | Success |
| PC-A | https://209.165.201.1 | HTTPS | Failure |
| PC-A | 209.165.201.1 | SSH | Failure |
| PC-B | 209.165.201.1 | SSH | Success |

If you get different results, double check your ACL configuration and application.

Step 3: Configure NAT.

The decision has been made that the entire organization should be using addresses in the 10.0.0.0/8 network  space. R1’s LAN is out of compliance. There are applications and services running in the R1 LAN that cannot  have their IP address changed without the entire system being rebuilt, so NAT is in order. Here are the  configuration tasks at R1:

| Task | Specification | Points |
| --- | --- | --- |
| Remove 192.168.1.0/24 from OSPF | Remove the appropriate network statement at R1 | 2 points |
| Create an ACL to identify hosts allowed to be translated | Create an ACL that matches the 192.168.1.0 network | 2 points |
| Configure Port Address Translation on the outside interface of R1 | Configure the NAT association between the ACL and the interface g0/0/0 so that it uses port address translation | 2 points |
| Identify the interfaces involved in NAT | Specify inside or outside on the appropriate interfaces | 2 points |

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 6 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

Step 4: Backup all device configurations.

| Task | Specification | Points |
| --- | --- | --- |
| Using the TFTP server on PC-B, backup the running configuration of all of your devices to PC-B using the TFTP protocol |  | 2 points |

Points for Step 1 (8 points):

Enter score here.

Points for Step 2 (12 points):

Enter score here.

Points for Step 3 (8 points):

Enter score here.

Points for Step 4 (2 points):

Enter score here.

Instructor Sign-off Part 4:

Instructor Sign-off

Total points (30 points):

Enter score here.

Part 5: Cleanup

NOTE: DO NOT PROCEED WITH CLEANUP UNTIL YOUR INSTRUCTOR HAS GRADED YOUR SKILLS  EXAM AND HAS INFORMED YOU THAT YOU MAY BEGIN CLEANUP.

Unless directed otherwise by the instructor, restore host computer network connectivity, and then turn off  power to the host computers.

Before turning off power to the router and switch, remove the NVRAM configuration files (if saved) from both  devices.

Disconnect and neatly put away all LAN cables that were used in the Final.

Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 7 of 8 www.netacad.com

CCNAv7 ENSA Skills Assessment

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many  interfaces the router has. There is no way to effectively list all the combinations of configurations for each router  class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device.  The table does not include any other type of interface, even though a specific router may contain one. An  example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be  used in Cisco IOS commands to represent the interface.

End of document

♥ 2019 - 2020 Cisco and/or its affiliates. All rights reserved. Cisco Public Page 8 of 8 www.netacad.com