BMIT3084 Enterprise Networking

Skills Based Assessment SBA Trial Exam

Topology – As shown in the Packet Tracer

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP | G0/1 | 208.165.103.1 | 255.255.255.0 | N/A |
|  | S0/0/0 (DCE) | 208.165.100.5 | 255.255.255.252 | N/A |
|  | S0/0/1 (DCE) | 208.165.100.1 | 255.255.255.252 | N/A |
|  | S0/1/0 (DCE) | 208.165.100.9 | 255.255.255.252 | N/A |
| VPN1 | S0/0/0 | 208.165.100.6 | 255.255.255.252 | N/A |
|  | G0/1 | 10.10.10.1 | 255.255.255.0 | N/A |
| VPN2 | S0/0/0 | 208.165.100.10 | 255.255.255.252 | N/A |
|  | G0/1 | 10.10.10.2 | 255.255.255.0 | N/A |
| NAT | S0/0/1 | 208.165.100.2 | 255.255.255.252 | N/A |
|  | G0/0 | 172.16.20.1 | 255.255.255.0 | N/A |
|  | G0/1 | 172.16.30.1 | 255.255.255.0 | N/A |
| DHCP | G0/0 | 172.16.20.2 | 255.255.255.0 | N/A |
|  | G0/1 | 172.16.10.1 | 255.255.255.0 | N/A |
| PC-A | NIC | DHCP Client |  |  |
| PC-Office | NIC | 172.16.10.128 | 255.255.255.0 | 172.16.10.1 |
| PC-B | NIC | DHCP Client |  |  |
| PC-C | NIC | 10.10.10.12 | 255.255.255.0 | 10.10.10.2 |
| PC-D | NIC | 10.10.10.129 | 255.255.255.0 | 10.10.10.1 |
| LocalServer/DNS Server | NIC | 172.16.30.254 | 255.255.255.0 | 172.16.30.1 |
| ExternalServer | NIC | 208.165.103.254 | 255.255.255.0 | 208.165.103.1 |
| ExternalHost | NIC | 208.165.103.203 | 255.255.255.0 | 208.165.103.1 |

Assessment Objectives

∙        Configure OSPF and static route

∙        Configure IP services: NAT and DHCP

∙        Configure GRE tunnel

∙        Configure network access control using ACLs

∙       Configure network management such as CDP, NTP and Syslog

User Profile

You are required to fill in your name and email address in the Packet Tracer User Profile

Pre-configuration:

All devices have been pre-configured with the basic IP addresses except PC-A, PC-B, PC-C and PC-D.

Enable password for NAT router is cisco

Complete the following task:

1.      Configure OSPF

a.      Configure a single area OSPF on NAT and DHCP routers using process-id 10 and area 0.

b.    Configuring the LAN interfaces as passive interfaces to ensure they do not form a neighbour relationship with each other.

c.     Adjust the reference bandwidth to 100000 on each OSPF router

d.      Propagate a default route configured using next hop IP address.

NAT: (9marks - 9%) ip route 0.0.0.0 0.0.0.0 208.165.100.1 router ospf 10 network 172.16.30.0 0.0.0.255 area 0 network 172.16.20.0 0.0.0.255 area 0 passive-interface g0/1 passive-interface s0/0/1 defaut-information originate auto-cost reference-bandwidth 100000 DHCP: (9marks - 18%) router ospf 10 network 172.16.20.0 0.0.0.255 area 0 network 172.16.10.0 0.0.0.255 area 0 passive-interface g0/1 auto-cost reference-bandwidth 100000

2.      Configure Static Route

a.      Configure a static route on ISP router to route to the public network address for Network CK1 and Kong2 using next hop IP address.

b.      Configure 2 static routes on ISP to route packets to Network Jessie3 using respective next hop IP address.

c.     Configure default static routes in VPN1, VPN2 and NAT routers to forward traffic to ISP using respective next hop IP address.

(7 marks - 25%) ISP: ip route 208.165.102.0 255.255.255.248 208.165.100.2 ip route 10.10.10.0 255.255.255.0 208.165.100.6 ip route 10.10.10.0 255.255.255.0 208.165.100.10 VPN1: ip route 0.0.0.0 0.0.0.0 208.165.100.5 VPN2: ip route 0.0.0.0 0.0.0.0 208.165.100.9

3.      Configure DHCP

a.      Network CK1

i.     Exclude first 5 IP addresses from the pool

ii.     DHCP pool name: POOL-CK1

iii.     DNS 172.16.30.254

b.      Network Kong2

i.     Exclude first 5 IP addresses from the pool

ii.     DHCP pool name: POOL-Kong2

iii.     DNS 172.16.30.254

(15 marks - 40%) DHCP: ip dhcp excluded-address 172.16.10.1 172.16.10.5 ip dhcp pool POOL-CK1 network 172.16.10.0 255.255.255.0 default-router 172.16.10.1 dns-server 172.16.30.254 ip dhcp excluded-address 172.16.30.1 172.16.30.5 ip dhcp pool POOL-Kong2 network 172.16.30.0 255.255.255.0 default-router 172.16.30.1 dns-server 172.16.30.254 NAT: int g0/1 ip helper-address 172.16.20.2 PC-B & PC-A to DHCP Client (3 marks - 43%) PC-C & PC-D configure static ip (9 marks - 52%)

4.      Configure NAT

a.      Configure dynamic NAT with Overload

i.     NAT pool name: NAT-TARUCPOOL

ii.     Public network address: 208.165.102.0/29

iii.     Reserve the LAST usable IP address for STATIC mapping to the LocalServer

iv.     Network Kong2 and Network CK1 are allowed to access to anywhere in the Internet (use keyword) using a named extended ACL NAT-ACL

b.      Configure static NAT

i.     Map the LocalServer to the LAST usable IP address from the 208.165.102.0/29

(8marks - 60%) NAT: ip nat pool NAT-TARUCPOOL 208.165.102.1 208.165.102.5 netmask 255.255.255.248 ip access-list extended NAT-ACL permit tcp 172.16.10.0 0.0.0.255 any eq www permit tcp 172.16.30.0 0.0.0.255 any eq www ip nat inside source static 172.16.30.254 208.165.102.6 ip nat inside source list NAT-ACL pool NAT-TARUCPOOL overload int g0/1 ip nat inside int g0/0 ip nat inside int s0/0/1 ip nat outside

5.      Configure a GRE tunnel between VPN1 and NAT router

a.      Tunnel number 1

b.      Use the FIRST usable IP address from 172.16.1.0/30 for the tunnel interface at VPN1

c.    In VPN1, configure a static route over the tunnel to the inside summarized network space 172.16.0.0 /16 using next-hop ip address.

d.    In NAT, configure a static route over the tunnel to VPN1 network 10.10.10.0 /24  using next-hop ip address.

(6marks - 66%) VPN1: int tunnel1 ip add 172.16.1.1 255.255.255.252 tunnel source s0/0/0 tunnel dest 208.165.100.2 ip route 172.16.0.0 255.255.0.0 172.16.1.2 (5marks - 71%) NAT: int tunnel1 ip add 172.16.1.2 255.255.255.252 tunnel source s0/0/1 tunnel dest 208.165.100.6 ip route 10.10.10.0 255.255.255.0 172.16.1.1

6.      Configure a GRE tunnel between VPN2 and NAT router

a.      Tunnel number 2

b.      Use the FIRST usable IP address from 172.16.2.0/30 for the tunnel interface at VPN2

c.    In VPN2, configure a static route over the tunnel to the inside summarized network space 172.16.0.0 /16 using next-hop ip address.

d.    In NAT, configure a static route over the tunnel to VPN2 network 10.10.10.0 /24  using next-hop ip address.

(6marks - 77%) VPN2: int tunnel2 ip add 172.16.2.1 255.255.255.252 tunnel source s0/0/0 tunnel dest 208.165.100.2 ip route 172.16.0.0 255.255.0.0 172.16.2.2 (6marks - 83%) NAT: int tunnel 2 ip add 172.16.2.2 255.255.255.252 tunnel sour s0/0/1 tunnel dest 208.165.100.2 ip route 10.10.10.0 255.255.255.0 172.16.2.1

7.      Configure named standard ACL (use keyword host wherever applicable)

a.      Configure a named standard ACL OFFICE-ACL on NAT router to allow Telnet access

b.      Use telnet password cisco

c.      Allow only PC-Office.

(4marks - 87%) NAT: ip access-list standard OFFICE-ACL permit host 172.16.10.128 line vty 0 4 password cisco login transport input telnet access-class OFFICE-ACL in

8.   Configure numbered extended ACL (use keyword host wherever applicable)

a.      Configure a numbered extended ACL using 188 on DHCP router

b.      Permit any DHCP clients communicate to any DHCP Servers

c.       Permit DNS access from Network CK1 to the DNS Server

d.       SECOND half of Network CK1 is allowed to HTTPS to the LocalServer (use port number)

e.      PC-Office is allowed to HTTPS to the LocalServer(use port number)

f.       SECOND half of the Network CK1 is allowed to FTP to the LocalServer

g.      Network CK1 is allowed to access 208.165.103.0/24

h.      Ensure PC-Office can telnet to G0/0 of NAT router

(1 marks - 88%) DHCP: access-list 188 permit udp any eq 68 any eq 67 (gbt answer) access-list 188 permit udp any any eq 67 access-list 188 permit udp any any eq 68 access-list 188 permit udp 172.16.10.0 0.0.0.255 host 172.16.30.254 eq domain access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 443 access-list 188 permit tcp host 172.16.10.128 host 172.16.30.254 eq 443 access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 20 access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 21 access-list 188 permit ip 172.16.10.0 0.0.0.255 208.165.103.0 0.0.0.255 access-list 188 permit tcp host 172.16.10.128 host 172.16.20.1 eq 23 int g0/1 ip access-group 188 in !junjun here got two marks access-list 188 permit udp any any eq 67 access-list 188 permit udp any any eq 68 access-list 188 permit udp 172.16.10.0 0.0.0.255 host 172.16.30.254 eq 53 access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 443 access-list 188 permit tcp host 172.16.10.128 host 172.16.30.254 eq 443 access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 20 access-list 188 permit tcp 172.16.10.128 0.0.0.127 host 172.16.30.254 eq 21 access-list 188 permit ip 172.16.10.0 0.0.0.255 208.165.103.0 0.0.0.255 access-list 188 permit tcp host 172.16.10.128 host 172.16.20.1 eq 23 int g0/1 ip access-group 188 in

9.   Configure Network Management

a. Configure NAT to get the date and time from 172.16.30.254 (LocalServer/ DNS Server).

b. Deactivate CDP for VPN1 at the global configuration mode.

c. Deactivate CDP for VPN2 at S0/0/0

d. Enable timestamp service in NAT for logging in 172.16.30.254 (LocalServer/ DNS Server).

Configure NAT to send Syslog messages to the Syslog server. The IP address of the Syslog server is 172.16.30.254 (LocalServer/ DNS Server).

NAT: ntp server 172.16.30.254 (2 marks - 90%) service timestamps log datetime msec (2 marks - 94%) (1 marks - 95%) service timestamps debug datetime msec logging on logging 172.16.30.254 (mark add at here) (1 marks - 91%) VPN1: no cdp run (1 marks - 92%) VPN2: int s0/0/0 no cdp ena

10. You are required to submit (upload and turn in) the following:

a.      A screen shot of the score with your User Profile details

b.      A configured Packet Tracer file.

~~~~~~~~~~~~~~~~~~~~~~~~~~ The End ~~~~~~~~~~~~~~~~~~~~~~~~~~