BMIT3084 Enterprise Networking

Skills Based Assessment SBA Trial Exam2

Duration: 1 hours 30 minutes

************************************************************************

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP | G0/0/0 | 209.165.202.1 | 255.255.255.0 | N/A |
|  | G0/0/1 | 209.165.201.1 | 255.255.255.0 | N/A |
|  | S0/2/0 (DCE) | 209.165.200.1 | 255.255.255.224 | N/A |
|  | S0/2/1 (DCE) | 209.165.204.1 | 255.255.255.252 | N/A |
| IT | G0/0/0 | 10.10.10.1 | 255.255.255.0 | N/A |
|  | G0/0/1 | 10.10.20.1 | 255.255.255.0 | N/A |
|  | S0/1/0 (DCE) | 192.168.1.9 | 255.255.255.252 | N/A |
|  | S0/1/1 (DCE) Backup link | 192.168.1.5 | 255.255.255.252 | N/A |
|  | S0/2/0 | 209.165.200.2 | 255.255.255.224 | N/A |
|  | Tunnel 3 | 172.16.2.25 | 255.255.255.252 | N/A |
| BAC | G0/0/0 | 10.10.10.2 | 255.255.255.0 | N/A |
|  | G0/0/1 | 10.10.30.1 | 255.255.255.0 | N/A |
| BR | G0/0/0 | 10.10.51.1 | 255.255.255.0 | N/A |
|  | G0/0/1 | 10.10.50.1 | 255.255.255.0 | N/A |
|  | S0/1/0 | 192.168.1.10 | 255.255.255.252 | N/A |
|  | S0/1/1 Backup link | 192.168.1.6 | 255.255.255.252 | N/A |
| BR2 | G0/0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
|  | S0/2/1 | 209.165.204.2 | 255.255.255.252 | N/A |
|  | Tunnel 3 | 172.16.2.26 | 255.255.255.252 | N/A |
| IT-LAN-DHCP-Client | NIC | DHCP Client |  |  |
| ADMIN-IT | NIC | 10.10.20.20 | 255.255.255.0 | 10.10.20.1 |
| IT SERVER | NIC | 10.10.10.10 | 255.255.255.0 | 10.10.10.1 |
| BRANCH-SERVER | NIC | 10.10.51.10 | 255.255.255.0 | 10.10.51.1 |
| BR-LAN1-PC | NIC | 10.10.51.20 | 255.255.255.0 | 10.10.51.1 |
| BR-LAN2-PC128 | NIC | 10.10.50.128 | 255.255.255.0 | 10.10.50.1 |
| BR-LAN2-DHCP Client | NIC | DHCP Client |  |  |
| BAC-LAN-PC31 | NIC | 10.10.30.31 | 255.255.255.0 | 10.10.30.1 |
| BAC-LAN-PC32 | NIC | 10.10.30.32 | 255.255.255.0 | 10.10.30.1 |
| TAR-UMT | NIC | 10.10.30.20 | 255.255.255.0 | 10.10.30.1 |
| BR2-PC | NIC | 172.16.1.20 | 255.255.255.0 | 172.16.1.1 |
| WEB-SERVER | NIC | 209.165.202.10 | 255.255.255.0 | 209.165.202.1 |
| OUTSIDE-HOST | NIC | 209.165.201.20 | 255.255.255.0 | 209.165.201.1 |

Assessment Objectives

1) Configure single area OSPFv2

2) Configure static routes and floating static route

3) Configure DHCP

4) Configure IP services: NAT

5) Configure GRE VPN tunnel

6) Configure network access control using ACLs

7) Configure Network management: Backup an IOS image file and LLDP

User Profile

You are required to fill in your name, email address and tutorial group number in the Packet Tracer User Profile

Pre-configuration:

a.     All devices have been pre-configured with the required IP addresses except IT-LAN-DHCP-Client and BR-LAN2-DHCP-Client.

b.     The enable password for BAC router is classSBA

*PART 1): Configure single area OSPFv2 on IT, BR and BAC routers.

| Task 1a) | Specification |
| --- | --- |
| On router | IT |
| Configure single-area OSPFv2 routing process | Use process id 67 and area 0 |
| Manually configure the router id | Use 1.1.1.1 for IT Router |
| Configure network statements | Configure a network statement for each locally attached network using a wild card mask that matches each network subnet mask. Note: 5m each will be deducted for any extra network statements |
| Default static route | Propagate a default static router in the OSPF routing updates (when necessary) |
| Configure passive interfaces | Configure all interfaces that are not directly connected to an OSPF neighbor to be passive |
| Tune the default value to make IT the Designated Router (DR). | Configure interface priority to 5 times more than the default value. |
| Tune the timers for your network | Configure the followings: a) hello interval timers to 20 seconds b) dead interval timers to 80 seconds respectively on the serial link to IT router |

Router IT: router ospf 67 router-id 1.1.1.1 (1m) network 10.10.20.0 0.0.0.255 area 0 (2m) network 10.10.10.0 0.0.0.255 area 0 (1m) network 192.168.1.8 0.0.0.3 area 0 (1m) default-information originate (1m) passive-interface g0/0/1 (2m) passive-interface s0/2/0 ip route 0.0.0.0 0.0.0.0 209.165.200.1 (1m) *Only configure on the interface that talking for ospf **Question says only configure this two on “serial” link interface s0/1/0 ip ospf hello-interval 20 (1m) ip ospf dead-interval 80 (1m) interface s0/1/1 ip ospf hello-interval 20 ip ospf dead-interval 80 *DR Selection only exceed on multi-access network interface g0/0/0 ip ospf priority 5 (1m)

| Task 1b) | Specification |
| --- | --- |
| On router | BR |
| Configure single-area OSPFv2 routing process | Use process id 67 and area 0 |
| Manually configure the router id | Use 2.2.2.2 for BR Router |
| Configure network statements | Configure OSPF routing using interface IP address and quad-zero masks Note: 5m each will be deducted for any extra network statements |
| Configure passive interfaces | Configure all interfaces that are not directly connected to an OSPF neighbor to be passive |
| Tune the timers for your network | Configure the followings: a) hello interval timers to 20 seconds b) dead interval timers to 80 seconds respectively on the serial link to BR router |

Router BR: router ospf 67 router-id 2.2.2.2 (1m) *Using IP address and quad-zero masks = use the interface IP address that link to the subnet network 192.168.1.10 0.0.0.0 area 0 (2m) network 10.10.50.1 0.0.0.0 area 0 (1m) network 10.10.51.1 0.0.0.0 area 0 (1m) passive-interface g0/0/0 (1m) passive-interface g0/0/1 (1m) **Question says only configure this two on “serial” link interface s0/1/0 ip ospf hello-interval 20 (1m) ip ospf dead-interval 80 (1m) interface s0/1/1 ip ospf hello-interval 20 ip ospf dead-interval 80

| Task 1c) | Specification |
| --- | --- |
| On router | BAC |
| Configure single-area OSPFv2 routing process | Configure OSPF routing on router interface(s) Use process id 67 and area 0 |
| Manually configure the router id | Use 3.3.3.3 for BR Router |
| Configure passive interfaces | Configure all interfaces that are not directly connected to an OSPF neighbor to be passive |

Router BAC: router ospf 67 router-id 3.3.3.3 (1m) network 10.10.10.0 0.0.0.255 area 0 (no mark but need for success dhcp) network 10.10.30.0 0.0.0.255 area 0 (no mark but need for success dhcp) passive-interface G0/0/1

*PART 2): Configure Static Routes and Floating Static Route.

The serial link connecting IT S0/1/1 and BR s0/1/1 interfaces is used as a backup link using floating static route.

| Task 2a) | Specification |
| --- | --- |
| Configure directly connected floating static routes | On IT router Note: to reach BR-LAN1 and BR-LAN2 with an AD of 113 using backup link with exit interface. |
| Configure static route over the tunnel | On IT router tunnel to the BR2-LAN using next hop IP address |
| Configure default static route | On IT router using next hop IP address to the Internet |

.1.4 backup .1.8 main route BR s0/1/1 .1.6 BR s0/1/0 .1.10 Router IT: ip route 10.10.51.0 255.255.255.0 s0/1/0 ip route 10.10.51.0 255.255.255.0 s0/1/1 113 (1m) ip route 10.10.50.0 255.255.255.0 s0/1/0 ip route 10.10.50.0 255.255.255.0 s0/1/1 113 (1m) ip route 172.16.1.0 255.255.255.0 172.16.2.26 (1m) interface tunnel3 ip add 172.16.2.25 255.255.255.252 (1m) tunnel source s0/2/0 (2m) tunnel dest 209.165.204.2 (1m) Router BR2: interface tunnel3 ip add 172.16.2.26 255.255.255.252 (1m) tunnel source s0/2/1 (1m) tunnel dest 209.165.200.2 (1m)

| Task 2b) | Specification |
| --- | --- |
| Configure directly connected default static route | On BR2 router to ISP router using exit interface |
| Configure static route over the tunnel | On BR2 over the tunnel to the inside summarized network space 10.10.0.0/16 using next hop IP address. |

Router BR2: ip route 10.10.0.0 255.255.0.0 172.16.2.25 (2m) ip route 0.0.0.0 0.0.0.0 s0/2/1 (1m)

| Task 2c) | Specification |
| --- | --- |
| Configure Floating default static route | On BR router Note: to reach ISP with an AD of 113 using backup link with exit interface. |

Router BR: ip route 0.0.0.0 0.0.0.0 s0/1/0 ip route 0.0.0.0 0.0.0.0 s0/1/1 113 (1m)

| Task 2d) | Specification |
| --- | --- |
| Configure directly connected static route | On ISP router using exit interface to the public network address space 209.165.203.0/30 of the inside network. |

Router ISP: ip route 209.165.203.0 255.255.255.252 s0/2/0 (1m)

Part 3): Configure DHCP on BAC router. Use helper-address 10.10.10.2 (if any)

| Task | Specifications |
| --- | --- |
| Configure DHCP server at BAC router | BAC router IT-LAN Network i. Exclude first 5 IP addresses from the pool ii. DHCP pool name: IT-LAN iii. Configure the default router iv. DNS 10.10.10.2 |
|  | BR-LAN2 Network v. Exclude first 10 IP addresses from the pool vi. DHCP pool name: BR-LAN2 vii. Configure the default router viii. DNS 10.10.10.2 |
| Configure helper-address | Use helper-address of 10.10.10.2 (if any). **you need to determine which router need to configure DHCP relay agent (if any). |
| Check your DHCP settings | Verify that IT-LAN-DHCP-Client learned the IP address and the above parameters Verify that BR-LAN2-DHCP-Client learned the IP address and the above parameters Note: Troubleshoot if you failed to get the addresses as required. |

Router BAC: ip dhcp excluded-address 10.10.20.1 10.10.20.5 (1m) ip dhcp pool IT-LAN (1m) network 10.10.20.0 255.255.255.0 (4m) default-router 10.10.20.1 (1m) dns-server 10.10.10.2 (1m) ip dhcp excluded-address 10.10.50.1 10.10.50.10 (1m) ip dhcp pool BR-LAN2 (2m) network 10.10.50.0 255.255.255.0 (3m) default-router 10.10.50.1 (2m) dns-server 10.10.10.2 (2m) Router BR: interface g0/0/1 ip helper-address 10.10.10.2 (1m) Router IT: interface g0/0/1 ip helper-address 10.10.10.2 (1m)

*Part 4) : Configure IP services: Dynamic Network Address Translations with Port Address Translations (PAT)

| Task | Specification |
| --- | --- |
| Configure static NAT | At IT router Map the IT-SERVER inside address to the FIRST usable IP address from the 209.165.203.0/30 address space |
| Configure NAT pool | Create a NAT pool named IT to use the SECOND usable IP address from the 209.165.203.0/30 address space |
| Configure a named standard ACL | On IT router, name the standard ACL as IT-NAT |
| Configure a named standard ACL | All hosts from BR-LAN1 and IT-LAN are allowed to be translated by using a named standard ACL IT-NAT (specify the LANs in this order) |
| Configure NAT translation | Associating the ACL and Pool |
| Identify the interfaces involved in NAT | Specify inside or outside on the appropriate interfaces. |

Router IT: ip nat inside source static 10.10.10.10 209.165.203.1 (1m) ip nat pool IT 209.165.203.2 209.165.203.2 netmask 255.255.255.252 (1m) ip access-list standard IT-NAT permit 10.10.20.0 0.0.0.255 permit 10.10.51.0 0.0.0.255 ip nat inside source list IT-NAT pool IT overload (2m) int s0/2/0 ip nat outside (1m) int s0/1/0 ip nat inside (1m) int s0/1/1 ip nat inside (1m) int g0/0/1 ip nat inside (1m) int g0/0/0 ip nat inside (1m)

Part 5) : Configure GRE VPN tunnel between BR2 and IT routers.

| Task |  | Specification |
| --- | --- | --- |
|  | Configure Tunnel interface 3 | On BR2 router i. Configure tunnel interface 3 using the addressing shown in the addressing table ii. Use tunnel source port on BR2 router and the tunnel destination ip address on the IT router. |
|  | Configure Tunnel interface 3 | On IT router i. Configure tunnel interface 3 using the addressing shown in the addressing table ii. Use tunnel source port on IT router and the tunnel destination ip address on the BR2 router. |

Part 6): Configure network access control using Access Control Lists (ACLs)

| Task 6a) | Specification |
| --- | --- |
| ACL | Use keyword host whenever applicable on BAC router |
| Configure a named standard ACL | TAR-UMT-TELNET |
| Telnet password | classSBA |
| Allow traffic | Allow only TAR-UMT to telnet to BAC router |

BAC Router: ip access-list standard TAR-UMT-TELNET (1m) permit host 10.10.30.20 line vty 0 4 password classSBA (2m) login access-class TAR-UMT-TELNET in (1m)

| Task 6b) | Specification |
| --- | --- |
| ACL | Use keyword host whenever applicable on BAC router |
| Use extended ACL number 120 | Allow TAR-UMT and all other hosts to access the BRANCH-SERVER but block the FIRST 31 hosts on BAC-LAN from accessing the BRANCH-SERVER. Permit all other traffic. Use ACL number 120 |
| Apply the ACL | Filter traffic originating from respective router. |

BAC Router: access-list 120 deny ip 10.10.30.0 0.0.0.31 host 10.10.51.10 access-list 120 permit ip any host 10.10.51.10 access-list 120 permit ip any any int g0/0/1 ip access-group 120 in (1m)

| Task 6c) | Specification |
| --- | --- |
| Create an extended ACL on BR router | use keyword host or any wherever applicable) on BR router |
| Control HTTPS traffic | Deny SECOND half of BR-LAN2 HTTPS to the BRANCH-SERVER (use port number). |
| Permit traffic | All other traffic |
| ACL Number | 100 |
| Apply the ACL | Filter traffic originating from respective router. |

BR Router: access-list 100 deny tcp 10.10.50.128 0.0.0.127 host 10.10.51.10 eq 443 access-list 100 permit ip any any (1m) int g0/0/1 ip access-group 100 in (1m)

| Task 6d) | Specification |
| --- | --- |
| Configure a named extended ACL | Name the ACL as ADMIN-IT-PING on router IT |
| Deny traffic on router IT. | Prevent all hosts on IT-LAN ping to the IT-SERVER EXCEPT ADMIN-IT |
| Permit traffic | Permit all other traffic |
| Apply the ACL | Filter traffic originating from respective router. |

IT Router: ip access-list extended ADMIN-IT-PING permit icmp host 10.10.20.10 host 10.10.10.10 deny icmp 10.10.20.0 0.0.0.255 host 10.10.10.10 permit ip any any int g0/0/1 ip access-group ADMIN-IT-PING in (1m)

Part 7: Configure Network management

| Task 7a) | Specification |
| --- | --- |
| Configure CDP | Disable CDP on the BR router. |
| Configure LLDP | Enable LLDP globally on BR router. |
| Configure LLDP | On BR, configure the links to the switches (interface g0/0/1) to only receive LLDP messages. |
| Configure CDP | Disable CDP on the BR-LAN2 switch. |
| Configure LLDP | Enable LLDP on the BR-LAN2 switch. |
| Configure LLDP | On the BR-LAN2 switch, configure the links to the BR router (interface g0/1) to only send, not receive, LLDP messages. |
| Configure LLDP | Disable LLDP completely on the BR-LAN2 switch access ports (interface f0/1 and interface f0/10) that are in use. |

BR Router: no cdp run (1m) lldp run (2m) int g0/0/1 lldp receive no lldp transmit (1m) BR-LAN2 Switches: no cdp run (1m) lldp run (1m) int g0/1 lldp transmit no lldp receive (1m) int ran f0/1, f0/10 no lldp transmit (3m) no lldp receive (2m)

| Task 7b) | Specification |
| --- | --- |
| Backup an IOS image file on BR router to BRANCH-SERVER at 10.10.51.10 (TFTP server). | Backup an IOS image file (isr4300-universalk9.16.06.04.SPA.bin) on BR router to BRANCH-SERVER at 10.10.51.10 (TFTP server). |

BR Router: do sh flash copy flash:tftp <copy filename> 10.10.51.10 (1m)

Part 8: SUBMISSION

1. You are required to screen shot User Profile details + score
2. Submit the final packet tracer files and rename your file name to score-programme-group-(full name).
eg: 100% RIS2-G6 (Tan Jia Mei)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ The End ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~