# Topology

# R1

en

conf t

hostname R1

no ip domain lookup

line console 0

logging synchronous

end

int g0/0/1

ip add 10.53.0.1 255.255.255.0

no shut

int Loopback1

ip add 172.16.1.1 255.255.255.0

no shut

router ospf 56

router-id 1.1.1.1

network 10.53.0.0 0.0.0.255 area 0

network 172.16.1.0 0.0.0.255 area 0

end

sh ip ospf nei

sh ip route ospf

ping 192.168.1.1

traceroute 192.168.1.1

conf t

int g0/0/1

ip ospf 56 area 0

ip ospf priority 50

ip ospf hello-interval 30

ip ospf dead-interval 120

exit

ip route 0.0.0.0 0.0.0.0 Loopback1

router ospf 56

default-information originate

auto-cost reference-bandwidth 1000

end

clear ip ospf process

end

sh ip ospf int g0/0/1

# R2

en

conf t

hostname R2

no ip domain-lookup

logging synchronous

end

int g0/0/1

ip add 10.53.0.2 255.255.255.0

no shut

int Loopback1

ip add 192.168.1.1 255.255.255.0

no shut

exit

int Loopback1

ip ospf 56 area 0

router ospf 56

router-id 2.2.2.2

network 10.53.0.0 0.0.0.255 area 0

network 192.168.1.0 0.0.0.255 area 0

end

sh ip ospf nei

sh ip route ospf

conf t

int Loopback 1

ip ospf 56 area 0

int g0/0/1

ip ospf hello-interval 30

int loopback 1

ip ospf network point-to-point

exit

router ospf 56

passive-interface Loopback1

auto-cost reference-bandwidth 1000

clear ip ospf process

end

sh ip route ospf

ping 172.16.1.1

traceroute 172.16.1.1

# S1

en

conf t

hostname S1

no ip domain-lookup

# S2

en

conf t

hostname S2

no ip domain-lookup

# EIGRP vs. OSPF

| EIGRP(Cisco Propie) | OSPF |
| --- | --- |
| router eigrp ? router-id ? network <network address> <willcart mask> passive-interface <> no auto-summary redistribute static | router ospf ? router-id ? network <network address> <willcart mask> area 0 passive-interface <> default-information originate |
|  | Additional of OSPF int g0/0/0 ip ospf <process id> area 0 ip ospf cost ? ip ospf priority ? ip ospf hello-interval ? ip ospf dead-interval ? |

*passive interface = do not share router information to the interface

OSPF very smart

Select one DR(Designated router) as monitor

Other one become BDR (backup )

Will share to DR then it compile (no need everyone listen each other, busy busy)