1. Assign IP address for the router interfaces and bring it up.

R1 interface s0/1/0 ip address 210.10.10.2 255.255.255.248 no shut interface g0/0/1 ip address 192.168.9.1 255.255.255.252 no shut R2 interface g0/0/1 ip address 192.168.9.2 255.255.255.252 no shut interface g0/0/0 ip address 192.168.8.193 255.255.255.192 no shut ISP interface s0/1/0 ip address 210.10.10.1 255.255.255.248 no shut

2. Configure VLANs in S1. VLAN 10 name is Subnet A .

VLAN 20 name is Subnet B. VLAN 30 name is Subnet C .

vlan 10 name Subnet_A vlan 20 name Subnet_B vlan 30 name Subnet_C interface f0/6 switchport mode access switchport access vlan 10 interface f0/7 switchport mode access switchport access vlan 20 interface range g0/23-24 switchport mode access switchport access vlan 30 interface f0/3 switchport mode trunk switchport trunk allowed vlan 10,20,30

3. Configure Router-on-a-stick in R1

interface g0/0/0.10 encapsulation dot1q 10 ip address 192.168.8.1 255.255.255.192 interface g0/0/0/20 encapsulation dot1q 20 ip address 192.168.8.65 255.255.255.192 interface g0/0/0.30 encapsulation dot1q 30 ip address 192.168.8.129 255.255.255.192 interface g0/0/0 no shut

4. Configure default route in R1 using exit interface.

Also configure static routes in ISP using exit interface.

R1 ip route 0.0.0.0 0.0.0.0 s0/1/0 ISP ip route 192.168.8.0 255.255.255.0 s0/1/0 ip route 192.168.8.0 255.255.255.192 s0/1/0 ip route 192.168.8.64 255.255.255.192 s0/1/0 ip route 192.168.8.128 255.255.255.192 s0/1/0 ip route 192.168.8.192 255.255.255.192 s0/1/0

5. Configure OSPF in R1 and R2 using network command.

Propogate the default route to R2 using OSPF.

Process ID: 8, Area: 0

R1 router ospf 8 network 210.10.10.2 0.0.0.7 area 0 network 192.168.9.0 0.0.0.3 area 0 network 192.168.8.0 0.0.0.127 area 0 network 192.168.8.0 0.0.0.63 area 0 network 192.168.8.64 0.0.0.63 area 0 network 192.168.8.128 0.0.0.63 area 0 passive-interface g0/0/0 passive-interface s0/1/0 default-information originate R2 router ospf 8 network 192.168.9.0 0.0.0.3 area 0 network 192.168.8.192 0.0.0.63 area 0 passive-interface g0/0/0

6. Configure DHCP in R1

* Exclude first 6 usable ip addresses for each subnets

* use the first address in each subnet as the

default-gateway address

* Poolname SUBNET_A is using 192.168.8.0/26

* Poolname SUBNET_B is using 192.168.8.64/26

* Poolname SUBNET_C is using 192.168.8.128/26

* Poolname SUBNET_D is using 192.168.8.192/26

* Configure IP helper address in R2

R1 ip dhcp excluded-address 192.168.8.1 192.168.8.6 ip dhcp excluded-address 192.168.8.65 192.168.8.70 ip dhcp excluded-address 192.168.8.129 192.168.8.134 ip dhcp excluded-address 192.168.8.193 192.168.8.198 ip dhcp pool Subnet_A network 192.168.8.0 255.255.255.192 default-router 192.168.8.1 ip dhcp pool Subnet_B network 192.168.8.64 255.255.255.192 default-router 192.168.8.65 ip dhcp pool Subnet_C network 192.168.8.128 255.255.255.192 default-router 192.168.8.129 ip dhcp pool Subnet_D network 192.168.8.192 255.255.255.192 default-router 192.168.8.193 R2 interface g0/0/0 ip helper-address 192.168.9.1

7. Configure PAT using outside interface in R1

access-list 1 permit 192.168.8.0 0.0.0.255 ip nat pool inside source list 1 interface s0/1/0 overload interface g0/0/0.10 ip nat inside interface g0/0/0.20 ip nat inside interface g0/0/0.30 ip nat inside interface g0/0/1 ip nat inside interface s0/1/0 ip nat outside

8. Configure Static NAT for WebServer in R1.

The public address is 210.10.10.6/29

ip nat inside source static 192.168.8.130 210.10.10.6